# Tool Guide – Traceroute

## Introduction

Traceroute is an essential tool for diagnosing network connectivity issues and performing network reconnaissance, as it maps the path packets take from your host to another host.

Safety Disclaimer: Use only on networks and systems for which you have explicit authorisation. Unauthorized use of tools can be Illegal, unethical and harmful.

## Setup and Launch

Step1: Open Deakin Detonator Toolkit

- Open Terminal in Linux

- Navigate to your Deakin Detonator Toolkit directory: (cd ~/Deakin-Detonator-Toolkit)

- Run DDT by using the following command line: (yarn run tauri dev)

Step2: Find Tool-example

- Click on Tools to the left (or use the quick search bar)

- If you clicked on Tools, scroll down and find Dig and click go

## Configuration and Usage

Step3: Configuring Traceroute

- Hostname/IP address: this fields sets the destination IP address

  - you can type either a domain name such as “example.com” or an IP address such as “8.8.8.8.”

- Traceroute custom (optional): you can customise traceroute with several commands, the list below is a set of examples (Courtesy of James Dunn)

  - "-m" Set the maximum number of hops to trace.

  - "-q" Sets the number of probe packets per hop.

  - "-w" Sets the wait time in seconds for each probe.

  - "-n" Tells Traceroute not to resolve hostnames.

  - "-f" Sets the starting hop number.

  - "-p" Sets the destination port for probes.

  - "-s" Specifies the source IP address to use.

  - "-4" or "-6" Forces the use of IPv4 or IPv6.

  - "-I" Use ICMP echo instead of the default UDP.

  - "-T" Use TCP SYN packets instead of the default UDP.

- Type of scan to perform: select the scan type you wish to use, the below are the available options

  - Traceroute ICMP scan

  - Traceroute TCP scan

  - Traceroute UDP scan

  - Traceroute custom scan

Step4: Results

- You will be given an output that should end with “process completed successfully” and be sure to scroll up in the Output field to see the rest.

- Optional: You can save your output by filling out the new “Output filename” field with an appropriate name (output.txt) and then clicking “Save Output To File” button

- Optional: You can copy your output to your clipboard by clicking the little clipboard in the top-right of the output area.

## Output Explanation

Note: your output will naturally be dependent on what domain/IP you checked and which options you selected, the following is non-comprehensive example output.